|  | B.N.M. Institute of Technology ##### An Autonomous Institution under VTU |
| --- | --- |

Report on Internship II (23ECE158)

DIGITAL LOCK SYSTEM WITH OTP VERIFICATION

Submitted in partial fulfillment of the requirements for the award of degree of

BACHELOR OF ENGINEERING

IN

ELECTRONICS AND COMMUNICATION ENGINEERING

Submitted by

Pavana P

1BG23EC077

Internship Carried Out

at

Cranes Varsity

Internal Guide 			         External Guide

Dr. Sameera P			         Aashrithest H G

Assistant Professor, 	                                VLSI Trainer

Dept. of ECE, BNMIT	                       Cranes Varsity

Department of Electronics & Communication Engineering

2025– 2026

B.N.M. Institute of Technology

An Autonomous Institution under VTU

DEPARTMENT: ELECTRONICS & COMMUNICATION ENGINEERING

CERTIFICATE

This is to certify that the Internship II (22ECE158) entitled Digital Lock System with OTP Verification carried out by Ms. Pavana P USN  1BG23EC077 a bonafide student of V Semester, in partial fulfillment for the Bachelor of Engineering in Electronics & Communication Engineering of the Visvesvaraya Technological University, Belagavi during the year 2024-25. It is certified that all corrections / suggestions indicated have been incorporated in the report. The internship report has been approved as it satisfies the academic requirements in respect of internship work prescribed for the said degree.

| Dr. Sameera P Assistant Professor, Dept. of ECE BNMIT, Bengaluru | Dr. Yasha Jyothi M Shirur HOD, Dept. of ECE BNMIT, Bengaluru | Dr. S.Y. Kulkarni Additional Director & Principal BNMIT, Bengaluru |
| --- | --- | --- |

DECLARATION

I, the undersigned solemnly declare that the report of the internship work entitled “Digital Lock System with OTP Verification” is based on my work carried out during the course study under the supervision of Dr. Sameera P,Assistant Professor, Department of ECE, BNM Institute of Technology, Bangalore and Aashrithest H G,VLSI Trainer,Cranes Varsity, Bangalore. This internship work was done at the Cranes Varsity, Bangalore.

I assert that the statements made and conclusions drawn are an outcome of the internship work. I further declare that, to the best of my knowledge and belief, that this report does not contain any work which has been submitted for the award of the degree or any other degree in this university or any other university.

Pavana P

DECLARATION

I, the undersigned solemnly declare that the report of the internship work entitled “………………….” is based on my work carried out during the course study under the supervision of           < Internal Guide Name>, < Designation >, Department of ECE, BNM Institute of Technology, Bangalore and < Industry Guide Name >, < Designation >, < Company Name >, Bangalore. This internship work was done at the < Company name with full details>, Bangalore.

I assert that the statements made and conclusions drawn are an outcome of the internship work. I further declare that, to the best of my knowledge and belief, that this report does not contain any work which has been submitted for the award of the degree or any other degree in this university or any other university.

<Student Name>

ACKNOWLEDGEMENTS

The satisfaction and euphoria that accompany the successful completion of the internship would be incomplete without mentioning the names of the people who made it possible. I am greatly indebted to our Director, Prof. T.J. Rama Murthy, Additional Director and Principal, Dr. S.Y. Kulkarni, Dean, Prof. Eishwar N. Maanay, Deputy Director, Dr. Krishnamurthy. G.N, and Head of Department, Electronics and Communication Engineering, Dr. Yasha Jyoti M Shirur, for giving me an opportunity to carry out the Internship. I am really indebted to our great institute BNMIT, which has provided me an opportunity to get a better degree in B.E. with the state-of-the-art facilities, and an impeccable environment.

It is with a deep sense of gratitude and great respect; I owe my indebtedness to my guide Dr. Sameera P. I thank him for his constant encouragement during the execution of this Internship. I also thank my internship coordinator Dr. Lakshmi Bhaskar / Prof. Sumathi A for making the conduction of this Internship successful. I express my heartfelt thanks to our Aashrithest H G at Cranes Varsity, for his guidance and unconditional support, despite of his busy work schedule. I thank my parents, without whom none of this would have been possible. Their patience and blessings have been with me at every step of this Internship. I would express my thanks to all my friends and all those who have helped me directly or indirectly for the successful completion of the Internship.

Pavana P

ABSTRACT

This report presents the design and verification of a secure Digital Lock System implementing two-factor authentication (2FA). The system's primary objective is to enhance security by requiring a sequential validation of a static 4-bit password and a simulated 4-bit One-Time Password (OTP). The core of the lock's control logic is a three-state Finite State Machine (FSM)—LOCKED, PWD_OK, and UNLOCKED—designed and implemented using Verilog HDL.

To ensure functional correctness, a comprehensive testbench was developed to simulate various operational scenarios, including correct and incorrect password/OTP entries. The design was compiled and rigorously verified using the EDA Playground online simulator. Analysis of the resulting waveforms and console logs confirms that the system operates as intended. The lock remains secure when presented with invalid credentials and transitions to the unlocked state only upon receiving the correct password followed by the correct OTP. The successful simulation validates the robustness and reliability of the FSM-based design, proving it meets all initial project objectives for a secure 2FA mechanism.

TABLE OF CONTENTS 						Pg. No.

- Chapter-1 – About the Company/ Organization…………

- Chapter-2 – About the Department……………………

- Chapter-3 – Project Details …………………………..

- Chapter-4    Results……………………………………

- Chapter-5 – Reflection Notes …………………………

References………………………………………………

LIST OF FIGURES

| Figure No. | Topic | Page No. |
| --- | --- | --- |
| 1 | Block diagram digital lock system with otp verification |  |
| 2 | Simulation Waveform results |  |

Chapter-01

About the Company/ Organization.

- Cranes Varsity is a pioneer Technical Training institute turned EdTech Platform offering Technology educational services for over 25 years

- Being a trusted partner of over 5000+ reputed Academia, Corporate & Defence Organizations we have successfully trained 1 Lakh+ engineers and placed 70,000+ engineers

- Cranes Varsity offers high-impact hands-on technology training to Graduates,  Universities, Working  Professionals, and the Corporate & Defence sectors.

- As trusted recruitment & Training partner we engage our Corporate through the “Hire, Train & Deploy” Model

- We stand by our principle – “We Assist Until We Place”- consistently strive for participants’ satisfaction and dedication to placement.

Cranes Varsity is a pioneer Technical Training institute turned EdTech Platform offering Technology educational services for over 25 years. A division of Cranes Software International Ltd, Cranes Varsity was established with an ambitious vision of bridging the gap between the technology academia and the industry.  The team continuously strives to be an organization that brings together technology and education, empowering aspiring professionals to seek assured placements and a lucrative career path.

Being a trusted partner of over 5000+ reputed Academia, Corporate & Defence Organizations we have successfully trained 1 Lakh+ engineers via its network of 2000+ Universities & colleges and placed 70,000+ engineers at major Indian Corporate & MNCs. Over 50,000+ Alumnae testifying our legacy and are the great ambassadors of Cranes Varsity Brand through their jobs worldwide.

Cranes Varsity carries a legacy of being the Authorized-training partner for Texas Instruments, MathWorks, Wind River & ARM. Cranes Varsity has training leadership in EMBEDDED, MATLAB & DSP, extending training domains to emerging industry trends like Automotive, IoT, VLSI, Java full-stack, Data Science & Business Analytics.

What We do

- Cranes Varsity offers training to Graduates – under the Finishing School Model,  Industry connects University programs, Upskilling programs for Working  Professionals, and Customized training to Corporate & Defence sectors.

- Cranes Varsity’s high-impact hands-on technology training catapults engineering students, graduates, and working professionals to be quickly employable in Niche high-end engineering fields.

- The in-house placement team further ensures that these students get placed in leading corporate firms – with whom Cranes Varsity has decades-old relationships. We stand by our principle – We Assist Until We Place.

- Being a trusted recruitment & training partner with Corporate, we engage with them for the “Hire, Train & Deploy” Model.

The Competitive Advantage is that

- Cranes Varsity offers an array of high-end technology training in Embedded  & Automotive Systems, C, C++, MATLAB, RTOS, Linux, LDD, BSP, Embedded  Testing, IoT Architecture, Protocols – Edge node Computing, Gateway &  Security with industrial IoT, DSP & MATLAB, VLSI design, Java technologies,  Cloud Computing, Azure, Python, Data Science & Analytics, Tableau, Artificial  Intelligence with Machine Learning, Deep Learning, NLP, Business Intelligence  and more

- Our Learning Approach Model is EEE – Educate, Evolve, Employment through our Pedagogical practices that integrate Learning Management Systems (LMSS). We continuously aim for our participants’ satisfaction and placement commitment through focused Training by our Subject-Matter Experts and Professionals.

- Chapter-02

System Verilog Verification

The escalating complexity of Very Large-Scale Integration (VLSI) circuits, driven by the relentless demand for high-performance, low-power semiconductor devices, has elevated functional verification to an indispensable discipline. With contemporary chips integrating billions of transistors, ensuring absolute functional correctness prior to fabrication is a paramount challenge. As manufacturing is an exceedingly expensive endeavor where post-fabrication bugs incur catastrophic financial and scheduling penalties, verification has become the most critical phase of the design cycle. SystemVerilog has decisively emerged as the industry's preeminent language for this task, extending Verilog with a powerful suite of verification capabilities including object-oriented programming, constrained random stimulus generation, assertions, and functional coverage, making it the definitive language for describing and validating complex digital systems.

The necessity of rigorous verification in VLSI is dictated by several uncompromising factors. The sheer intricacy of modern SoCs renders manual testing entirely impractical, necessitating the automation and precision that SystemVerilog provides. The economic imperative is stark; a single design flaw discovered post-fabrication can result in multi-million dollar losses. Furthermore, intense time-to-market pressures demand that bugs be detected and rectified early in the design flow. In safety-critical domains such as automotive, aerospace, and medical electronics, where system failure is not an option, robust verification ensures adherence to stringent safety and reliability standards. The adoption of standardized methodologies like the Universal Verification Methodology (UVM), built upon SystemVerilog, has solidified its role by enabling scalable and reusable verification infrastructures.

SystemVerilog provides an arsenal of advanced features that make verification both effective and scalable. Its cornerstone is constrained random verification, which autonomously generates a vast space of test vectors within user-defined constraints, thereby exposing obscure corner-case bugs. This is complemented by functional coverage, a powerful metric for quantifying how much of the design’s functionality has been exercised, ensuring no critical scenarios are overlooked. Assertions are employed to continuously monitor design properties and protocol rules during simulation, enabling the early detection of logical or functional mismatches. Its object-oriented programming paradigm facilitates the construction of modular, reusable, and highly sophisticated testbenches, while features like interfaces and clocking blocks streamline the management of signals and timing between the testbench and the design.

Modern VLSI verification follows structured methodologies in which SystemVerilog is central. While traditional directed testing remains applicable for simpler blocks, it lacks the scalability required for complex chips. Random stimulus testing, powered by SystemVerilog’s randomization features, excels at uncovering unforeseen corner cases. The prevailing methodology, Coverage-Driven Verification, synergizes random testing with functional coverage to ensure a thorough and intelligent exploration of the design space. The Universal Verification Methodology (UVM) represents the apex of this evolution. As a standard built on SystemVerilog, UVM provides a structured, layered testbench architecture comprising generators, drivers, monitors, and scoreboards, which facilitates the creation of powerful, reusable, and scalable verification environments.

A typical SystemVerilog testbench is a modular architecture of interacting components. A generator produces transaction-level stimulus, which a driver translates into pin-level signals for the design under test (DUT). A monitor observes the DUT’s outputs, converting them back into transactions for the scoreboard, where the actual results are meticulously compared against expected outcomes. Concurrently, functional coverage components measure the extent of the verification, and assertions validate protocol compliance and design constraints. This modular structure imparts flexibility and reusability, significantly reducing development effort across multiple projects. The advantages are numerous: a unified language for design and verification, scalability from IP blocks to full SoCs, and enhanced debugging through assertions and coverage reports, all fostering faster, more reliable verification cycles.

The applications of SystemVerilog verification are pervasive throughout the semiconductor industry. It is fundamental in processor and SoC verification for validating complex pipeline execution, cache coherence, and memory subsystem operations. High-speed communication protocols like PCIe, USB, and Ethernet depend on it to certify compliance with rigorous industry standards. Furthermore, it is crucial for verifying advanced low-power design features and is indispensable in safety-critical applications where zero-defect reliability is mandatory.

Despite its power, the discipline faces persistent challenges. The immense complexity of modern designs demands substantial simulation resources, often resulting in protracted verification cycles. Creating truly reusable verification IP within the UVM framework requires deep expertise. Moreover, engineers must possess sophisticated programming skills to fully leverage the object-oriented and constrained-random capabilities of the language.

Looking ahead, SystemVerilog verification is evolving in synergy with emerging technologies. Formal verification methods are being increasingly integrated with simulation to mathematically prove design properties. Machine learning is being explored to optimize test generation and predict error-prone design regions. Cloud-based platforms offer massively parallel simulation capabilities, promising to dramatically curtail verification timelines. The Portable Stimulus Standard (PSS) is also emerging as a complementary methodology for generating reusable test scenarios at a higher level of abstraction.

In conclusion, SystemVerilog has fundamentally redefined functional verification in VLSI design, offering a sophisticated framework to ensure the correctness of increasingly complex silicon. Through powerful methodologies like UVM, engineers can construct standardized, reusable testbenches that mitigate verification effort and accelerate time-to-market. As chip complexity continues its exponential trajectory, SystemVerilog verification will remain the indispensable cornerstone of the VLSI design cycle, adapting and integrating with new paradigms like machine learning and formal methods to meet the challenges of next-generation semiconductor development.

Chapter-03

Project Details

3.1 Objective

The primary and most fundamental objective of this project was to design a secure digital lock system based on the robust principle of two-factor authentication (2FA). This goal was established to address the inherent vulnerabilities of traditional single-factor systems, which rely solely on one credential, such as a password. A single point of failure, like a stolen password, compromises such systems entirely. Therefore, the project aimed to create a layered defense mechanism by requiring two distinct credentials: a static password, representing "something the user knows," and a simulated One-Time Password (OTP), representing "something the user has." The OTP, being dynamic and session-specific, provides a critical second layer of security. Even if the primary password were to be compromised, an unauthorized user would still be unable to gain access without the time-sensitive OTP, which would be delivered to a device in the legitimate user's possession. This design choice directly elevates the lock's security, ensuring that access is granted only after a more rigorous and reliable authentication process is successfully completed. The core of this objective was to build a system that is resilient to common attack vectors and upholds the principles of secure access control.

A second critical objective was to utilize a Finite State Machine (FSM) as the architectural core for the lock's control logic. This was a deliberate engineering decision, chosen because the lock's operation is inherently sequential and state-dependent. The system's response to any input is determined by its current state, which holds the memory of past events. An FSM provides a clear, predictable, and robust framework for managing this behavior. The design was structured around three distinct states: LOCKED, PWD_OK, and UNLOCKED. The LOCKED state serves as the secure default, awaiting the first authentication factor. Upon receiving a correct password, the system transitions to the intermediate PWD_OK state, a crucial step that logically separates the two authentication events and triggers the simulated otp_send signal. Only from this state can the system proceed to the UNLOCKED state, and only upon the validation of the correct OTP. This FSM-based approach enforces a strict, unalterable sequence of operations, making the system's logic transparent, manageable, and less prone to errors. By modeling the system with a finite number of states and explicit transitions, the design becomes deterministic and far easier to analyze and debug than less structured approaches.

The final, indispensable objective was to develop a comprehensive testbench to rigorously simulate and verify the design's functionality. In digital circuit design, verification is paramount; a design is considered flawed until proven otherwise. The purpose of the testbench was to act as an adversary, subjecting the digital lock to a battery of tests to confirm its correctness under all relevant conditions. This went beyond simply testing the ideal "happy path" where the user enters everything correctly. The test plan was meticulously crafted to include critical scenarios such as a system reset to ensure proper initialization, attempts with incorrect passwords to validate the first security layer, and multiple attempts with incorrect OTPs to confirm the second layer's integrity and its retry mechanism. The verification process relied on two key methods within the EDA Playground environment: detailed waveform analysis to visually inspect the behavior of every signal over time, and automated console checks using $display statements to log key events and confirm that checkpoints were passed successfully. This thorough verification process was not merely an add-on but a core objective, providing the necessary proof that the FSM logic was implemented correctly and that the security principles of the 2FA system were reliably upheld in practice.

3.2 Scope

The scope of the "Digital Lock System with OTP Verification" project is precisely defined to cover the design, simulation, and verification of the core digital logic for a two-factor authentication mechanism. The project is fundamentally a digital design exercise focused on creating a robust and logically sound security controller, rather than a complete physical system.

The primary component within the scope is the Finite State Machine (FSM) controller. This includes the complete logic for its three defined states: LOCKED, PWD_OK, and UNLOCKED. The scope covers all state transition logic, ensuring the system moves between these states only under the correct, predefined conditions. This involves the validation of a hardcoded, static 4-bit password and a subsequent hardcoded, 4-bit One-Time Password (OTP). The logic to handle both correct and incorrect credential entries is a central part of the design.

Furthermore, the scope includes the simulation and verification of this design. This was accomplished by creating a dedicated Verilog testbench responsible for generating the necessary stimuli (clock, reset, and input data sequences). The verification process specifically covers a set of critical scenarios: a successful unlocking sequence, an incorrect password attempt, and multiple incorrect OTP attempts to test the retry functionality. The analysis of the resulting digital waveform and console logs to confirm the circuit's behavior is the final component of the project's scope.

It is important to note what lies outside the scope of this project. The design does not include the actual hardware for an OTP generation or delivery system (e.g., a connection to a mobile network); this functionality is simulated by the otp_send signal. The project is also confined to the digital simulation environment of EDA Playground, meaning the physical implementation of the lock on hardware like an FPGA or ASIC is considered future work. Finally, advanced security features such as an attempt counter that would lock the system after too many failed tries, or an automatic re-locking timer, were not included in the scope of this core design.

3.3 Description

In an increasingly interconnected world, the need for robust digital security has become paramount. Traditional security measures, such as a single password, represent a single point of failure that can be easily compromised. To address this critical vulnerability, this project undertakes the design and verification of a Digital Lock System with OTP Verification, a modern security mechanism built upon the principle of two-factor authentication (2FA). This system enhances security by requiring two distinct forms of credentials before granting access: a static, pre-set password and a dynamic, session-specific One-Time Password (OTP). The project serves as a practical exploration of designing sequential, state-dependent logic using Verilog Hardware Description Language (HDL) and verifying its correctness through rigorous simulation.

The architectural heart of the digital lock is a Finite State Machine (FSM), a computational model perfectly suited for event-driven systems that transition through a well-defined series of states. The FSM acts as the central controller, intelligently managing the lock's behavior based on user inputs and its current operational state. The entire logic is encapsulated within three distinct states:

- LOCKED: This is the initial and default state of the system. Upon power-up or reset, the FSM enters this state, ensuring the lock is always in a secure condition. In the LOCKED state, the system is vigilant, waiting exclusively for the first authentication factor—the password. Any other inputs, such as an OTP entry, are disregarded. The primary output, lock_status, is held high (logic '1'), indicating that the system is secure and access is denied.

- PWD_OK: This is the intermediate state, which can only be reached after a valid password has been entered while the system was in the LOCKED state. The transition to this state signifies the successful completion of the first layer of security. Upon entering the PWD_OK state, the FSM performs a critical action: it asserts the otp_send output signal. In a real-world application, this signal would trigger a separate module to generate and send an OTP to the user's registered device (e.g., a mobile phone). In this simulation-focused project, this signal serves as a confirmation that the system is now ready to receive and validate the second authentication factor. The lock_status remains high, as the authentication process is not yet complete.

- UNLOCKED: This is the final, target state, representing the successful authentication of the user. The FSM transitions to this state only if the correct OTP is entered while the system is in the PWD_OK state. Upon entering the UNLOCKED state, the lock_status output is driven low (logic '0'), signifying that the lock is now open and access is granted. The system will remain in this state until it is reset.

The user interaction workflow is a clear, two-step process. First, the user presents the 4-bit password on the password_in data lines and asserts the enter_pwd signal. The FSM compares this input against the stored password. If they match, it transitions to PWD_OK. If not, it remains in the LOCKED state. Once in the PWD_OK state, the user presents the 4-bit OTP on the otp_in data lines and asserts the enter_otp signal. The FSM validates this input against the stored OTP. A correct OTP leads to the UNLOCKED state. An important feature of the design is its handling of incorrect OTPs; if an invalid OTP is entered, the FSM does not reset but remains in the PWD_OK state, allowing the user to retry without having to re-enter the password.

To prove the design's functional correctness, a comprehensive Verilog testbench was created. This testbench instantiates the digital lock module and methodically drives its inputs to simulate a realistic sequence of user actions. It begins by asserting the reset signal to ensure proper initialization. It then tests the system's resilience by providing an incorrect password, followed by the correct password. Subsequently, it tests the OTP validation logic by providing two consecutive incorrect OTPs before finally providing the correct one to unlock the system. Throughout this process, the testbench monitors the outputs and uses $display statements to print status messages to the simulation console, providing an automated and clear confirmation that the lock behaves exactly as intended at every step. The resulting waveform provides a visual, time-based confirmation of the system's flawless logic and robust security.

3.4 Methodology

The methodology for designing and verifying the Digital Lock System in this project follows a systematic approach that combines FSM design principles, Verilog implementation, and simulation on EDA Playground.

- Problem Definition :The first step is to clearly define the requirements of the secure lock. This includes establishing the two-factor authentication (2FA) process, where access is granted only after a correct static password is followed by a correct One-Time Password (OTP). For this project, a simplified system is considered, where both the password and OTP are 4-bits wide. The machine's behavior for both valid and invalid sequences of inputs is precisely defined to ensure it remains secure under all tested conditions.

- FSM State Diagram :A state diagram is drawn to represent the lock's sequential behavior. Three distinct states are identified: LOCKED, PWD_OK, and UNLOCKED. Transitions between these states are defined based on inputs. For example, entering the correct password in the LOCKED state transitions the system to PWD_OK. Subsequently, entering the correct OTP while in the PWD_OK state transitions the system to UNLOCKED. The diagram also specifies that incorrect inputs will not advance the state, ensuring the lock's integrity.

- Verilog Coding :The FSM is implemented using Verilog HDL. A state variable is defined using a reg data type, and a standard two-always block structure is used to describe the logic. A synchronous always @(posedge clk or posedge rst) block is used to capture the sequential logic for state updates. The state transition logic itself is implemented within a combinational always @(*) block using a case statement. The output logic (lock_status and otp_send) is also defined within this block to determine the system's outputs based on the current state.

- Testbench Development :A separate testbench module is written in Verilog to provide different input scenarios to the digital lock. The testbench simulates realistic conditions such as a system reset, entering an incorrect password, entering the correct password, providing multiple incorrect OTPs, and finally entering the correct OTP. The testbench also includes clock generation and applies these input stimuli over a defined timeline.

- Simulation on EDA Playground :The code for both the digital lock design and the testbench is uploaded and simulated on EDA Playground, an online platform for hardware simulation. Waveform analysis is performed by inspecting the VCD (Value Change Dump) file generated by the simulation. This allows for a visual check to confirm if the FSM transitions and output signals occur exactly as expected.

- Analysis and Verification :After simulation, the results are thoroughly analyzed to verify correctness. Any mismatches between the expected behavior (from the state diagram) and the observed outputs in the waveform are identified and debugged by reviewing the FSM logic and Verilog code. The console logs, which contain messages from the testbench, are also reviewed to provide an additional layer of confirmation that key verification points were passed successfully.

- Extensions and Improvements :Once the base design is verified, potential improvements are considered. These extensions, identified as future scope, could include implementing an attempt counter to prevent brute-force attacks after multiple failed entries, adding an automatic re-locking timer, or preparing the design for hardware synthesis to be implemented on an FPGA or ASIC. These extensions would involve adding new states and more complex logic to the FSM.

3.5 Block Diagram

FIGURE 1.Block diagram digital lock system with otp verification

The process begins with the system in a secure LOCKED state, awaiting user input. Upon password entry, the system validates the credential; an incorrect password is disregarded, keeping the lock secure. If correct, the system transitions to an intermediate PWD_OK state and simulates sending a One-Time Password. In this new state, the system waits for the second credential to be entered and validated. An incorrect OTP causes the system to remain in the PWD_OK state, allowing a retry. If the OTP is correct, the system moves to the final UNLOCKED state, setting the lock status to open and successfully concluding the authentication process.

Chapter-04

Results

The Digital Lock System with OTP Verification was successfully simulated and verified using the EDA Playground online platform. The results, obtained from both the simulation waveform and the console log output, provide conclusive evidence that the design is fully functional and meets all the specified objectives. The system correctly implements the two-factor authentication logic, remains secure against incorrect inputs, and transitions to the unlocked state only when the precise sequence of valid credentials is provided.

FIGURE 2.Simulation Waveform results

- 4.1  Simulation Waveform Analysis

- Initial Reset State (0-10 ns): At the beginning of the simulation, the rst signal is asserted high. The waveform clearly shows that this forces the lock_status output to a logic '1'. This confirms that the system correctly initializes into a secure, LOCKED state.

- Incorrect Password Attempt (~20-30 ns): The testbench first applies an incorrect password (4'b1111) and asserts the enter_pwd signal. As seen in the waveform, the lock_status remains high. This result is critical as it demonstrates that the first layer of security is effective and correctly rejects unauthorized access attempts.

- Correct Password Attempt (~40-50 ns): Next, the correct password (4'b1010) is applied. In response, the waveform shows a distinct, single-cycle pulse on the otp_send signal. This is a crucial result, as it visually confirms that the Finite State Machine (FSM) has successfully validated the password and transitioned from the LOCKED state to the PWD_OK state, ready to accept the OTP.

- Incorrect OTP Attempts (~60-90 ns): To test the second layer of security, the testbench applies two consecutive incorrect OTPs (4'b1111 and 4'b0011). The waveform shows that despite these inputs, the lock_status correctly remains at logic '1'. This verifies that the OTP validation logic is working and that the system allows for OTP retries without resetting, as intended by the design.

- Correct OTP and Successful Unlock (~100-110 ns): Finally, the testbench applies the correct OTP (4'b0101). The waveform shows the most important result of the simulation: the lock_status signal transitions from '1' to '0'. This demonstrates the successful completion of the two-factor authentication sequence and confirms that the FSM has transitioned to the UNLOCKED state.

4.2 Console Log Verification

The console output generated during the simulation provides a secondary, text-based confirmation that corroborates the waveform analysis. The testbench was programmed with $display statements to report on the status at critical checkpoints.

The following output was captured:

[70] Wrong OTP, still locked

[110] Lock unlocked successfully!

- The first message, [70] Wrong OTP, still locked, was printed at simulation time 70 ns. This aligns perfectly with the waveform, corresponding to the moment after the first incorrect OTP was entered, confirming the lock remained secure.

- The second message, [110] Lock unlocked successfully!, was printed at 110 ns. This corresponds exactly to the time in the waveform when the lock_status transitioned to '0' after the correct OTP was entered.

Chapter-05

5.1 Reflection Notes

This project, centered on the Digital Lock System with OTP Verification, was a pivotal learning experience during my internship at Cranes Varsity. It served as a practical application of the digital logic concepts I had studied, effectively bridging the gap between academic theory and the real-world challenges of VLSI design. The process of translating the abstract model of a Finite State Machine (FSM) into functional Verilog code was particularly insightful. Designing the state diagram forced me to think through every possible user scenario, and implementing the LOCKED, PWD_OK, and UNLOCKED states with precise transition logic made the theoretical concepts tangible. It underscored the importance of having a well-structured and logical plan before beginning to write any code.

The most demanding yet rewarding aspect of the project was undoubtedly the verification phase. My initial approach to creating a testbench was too simplistic, as I only tested the ideal successful unlocking sequence. I quickly learned that a design's robustness is only proven when it is challenged with incorrect and unexpected inputs. I encountered a significant bug where the FSM would get stuck in the PWD_OK state, and it was only through the meticulous analysis of the simulation waveform in EDA Playground that I could debug it. Tracing the signals visually allowed me to pinpoint the flaw in my combinational logic, a process that was an invaluable lesson in systematic problem-solving and highlighted the critical role that verification tools play in digital design.

Ultimately, this project has solidified my understanding of the entire digital design workflow, from initial specification and implementation to rigorous verification. It provided a holistic view, teaching me that designing a circuit and proving its correctness are two equally important halves of the same whole. The hands-on experience of writing both the synthesizable design and the procedural testbench has greatly enhanced my skills and confidence. I have moved from merely knowing about digital design to applying that knowledge to create a functional, secure system, which has sharpened my engineering approach and better prepared me for future challenges in the VLSI field.

5.2 References

P. S. Navale and S. R. Patil, “FPGA Based Digital Lock System Using RFID and Password,” International Journal of Advanced Research in Electrical, Electronics and Instrumentation Engineering, vol. 4, no. 5, pp. 4102–4106, 2015.

M. A. A. Rahman and Z. A. Zukarnain, “Implementation of One-Time Password Algorithm on Embedded System,” 2012 International Conference on Computer and Communication Engineering (ICCCE), IEEE, pp. 658–662, 2012.

A. K. Sah, A. Barik, and A. Pradhan, “Design and Implementation of Digital Number Lock System Using Finite State Machine,” International Journal of Engineering Research & Technology, vol. 6, no. 04, 2017.

K. Anvekar and S. Banakar, “A Verilog Implementation of a Digital Combination Lock based on a Finite State Machine,” International Journal of Computer Applications, vol. 118, no. 3, pp. 29–32, 2015.

R. S. S. Wu, M. F. A. Aziz, N. A. H. N. Azman, and S. K. Sahari, “GSM Based Home Security System Using Arduino,” 2017 IEEE 8th Control and System Graduate Research Colloquium (ICSGRC), IEEE, pp. 110–115, 2017.

A. A. Al-Azzawi, A. M. Al-Shayea, and A. H. Al-Dulaimi, “Design and Implementation of a Secure Smart Door Lock System Using FPGA,” International Journal of Interactive Mobile Technologies (iJIM), vol. 15, no. 13, pp. 58–71, 2021.

S. K. Sahoo and R. K. Sahu, “An Efficient FSM-based Digital Locking System Design and FPGA Implementation,” in Innovations in Electrical and Electronic Engineering, Singapore: Springer, 2021, pp. 627–636.

M. A. Hasan, M. S. Islam, and M. R. Islam, “Design and Implementation of an IoT-Based Secure Door Lock System with OTP Authentication Using FPGA,” 2022 IEEE International Conference on Consumer Electronics-Taiwan (ICCE-TW), IEEE, pp. 1–2, 2022.

R. Sharma and A. K. Singh, “A Novel Approach for Secure Keypad-Based Digital Lock System Using Verilog HDL,” Journal of Physics: Conference Series, vol. 1950, no. 1, p. 012059, 2021.